ECE0202: Embedded Systems and Interfacing

Lab 5: Stepper Motor Control (in assembly)

Objectives

- Use full stepping to control the speed and position of a stepper motor

- Generate pulse waveforms to control a stepper motor

- Use Darlington transistor arrays to perform high-current driving with extremely low input current

Pre-Lab Reading

- Read Textbook Chapter 16: Stepper Motors

- Watch the following video tutorials (credit to pcbheaven):

  - http://www.youtube.com/watch?v=MHdz3c6KLrg

  - http://www.youtube.com/watch?v=t-3VnLadIbc

- Answer the pre-lab questions

Deliverables

- (40 points) Basic requirement: Code that operates a simulated “windshield wiper” on the stepper motor.

  - When the on-board button(blue) is held down, the stepper motor will rotate approximately 145° counterclockwise, then 145° clockwise, then repeat over and over. When the button is released, the “windshield wiper” should finish its current routine and stop when it returns to 0°.

- (10 points) Answers to the pre-lab questions. Indicate how each group member participates and contributes to the lab at the end of the lab report.

- (50 points) A demonstration of the operational circuit to an instructor or TA.

Please submit your code and pre-lab answers and your Circuit photo as a pdf to Black board.

Stepper Motor Overview

| Motor model | 28BYJ-48 |  | Number of phases | 2 |  |
| --- | --- | --- | --- | --- | --- |
| Rated voltage | 5V DC |  | Geared reduction ratio | 1/64 |  |
| DC resistance per phase | 50Ω±7%(25℃) |  | Pull in torque | >300gf.cm / 5VDC 100pp |  |

image from forum.arduino.cc

The gear ratio is:

31×32×26×2211×10×9×9=63.68395

Full-stepping

- Internal motor: 32 steps per revolution

- Great reduction ratio: 1/63.68395, approximately 1/64

- So it takes 32×64=2048 steps per revolution for the output shaft

Half-stepping

- Internal motor: 64 steps per revolution

- Great reduction ratio: 1/63.68395 ≈ 1/64

- So it takes 64×64=4096 steps per revolution for the output shaft

Interfacing the stepper motor requires four pins. We select the following four pins to control the stepper motor: PB2, PB3, PB6, and PB7. The textbook provides a connection diagram for stepper motor Mabuchi #PF35T, which is very similar to the diagram below.

+

-

Pre-Lab Questions

- Fill out the following timing diagrams for the GPIO outputs that create a full-stepping sequence and a half-stepping sequence.

- How would you change your code to vary the rotation speed of the stepper motor?

- How would you reverse the rotation direction of the stepper motor?

Participation and Contribution

Please indicate the participation and contribution for each group member using the following table.

| Name | Participation and Contribution |
| --- | --- |